Configuring a USB Card Reader:

Step 1. Converting USB unit from Keyboard Emulation to HID mode:

- Install this application from the Magtek web site (self extracting zip) on the computer that will be used as a kiosk.

- When prompted to install source code - choose no.

- Run application with the USB card reader attached. The status bar at the bottom will say "detecting device" for a few moments, once the device has been detected, it will report the devices mode (keyboard emulation or HID)

- If it is in keyboard emulation mode, it will need to be converted to HID Mode: Click on ‘Load File’ button, double-click on the file (magtek application installation directory - ie C:\Program Files\Magtek)\Change To HID.txt.

- Then click on the ‘Download’ button (top right) or run the first two commands - 01 10 00, then 02 to rescan.

Step 2. Enabling Kiosk ActiveX:

- Login as an administrator into the computer that will be used as a kiosk.

- Open Edge and go to the following URL: https://apps.iu.edu/kpme-prd/kioskusb/

- You should be prompted to download and install the corresponding ActiveX controller, please do so.

Result: You should be able to see some feedback on the screen after swiping your Campus Id card. If you swipe the card of a current TIME employee, their timesheet should show up.

Step 3. Linking the new Kiosk Group Policy:

- Create a new Organizational Unit (OU)
Warning! If you have a heterogeneous environment with 64-bit and 32-bit versions of the browser, they must be in separate OUs

- Link the OU to the following existing Group Policy Object (GPO):

§  for production 64-bit: bl-uits-tk-prod-edge-64-kiosk

- Move the computer that will be used as a kiosk under this OU.

- Either wait for the GPO to be picked up or run the “gpupdate /force” command in the kiosk

Result:  When restarted, the computer should automatically open Edge and it should redirect users to the URL mentioned in 2b above.

Note: If you are configuring your own GPO, make sure to turn on compatibility view in Edge and enable it for all iu.edu sites.

Configuring a Proximity Reader:

Step 1. Install the drivers for the proximity reader:

- Plug the proximity reader into the USB port and login as an administrator into the computer that will be used as a kiosk.

- Open Edge or Chrome and go to the following URL: https://apps.iu.edu/kpme-prd/kiosktap

- Verify that the proximity reader is detected and sending data by attempting to login

Step 2. Linking to the new Kiosk Group Policy:

- Create a new Organizational Unit (OU)
Warning! If you have a heterogeneous environment with 64-bit and 32-bit versions of the browser, they must be in separate OUs

- Link the OU to the following existing Group Policy Object (GPO):

§  for production 64-bit: bl-uits-tk-prod-prox-64-kiosk

- Move the computer that will be used as a kiosk under this OU.

- Either wait for the GPO to be picked up or run the “gpupdate /force” command in the kiosk

Result:  When restarted, the computer should automatically open the web browser and it should redirect users to the URL mentioned in 1b above.